Top of Form

|  | Spartan Chemical Company, Inc. Material Safety Data Sheet |
| --- | --- |

SECTION I: PRODUCT INFORMATION

SECTION I: PRODUCT INFORMATION

| Product Name or Number (as it appears on label): | Product Division: |
| --- | --- |
| CUTTER EXP HW Product Number: 2925 | IPG |

| Spartan Chemical Company, Inc. | Product/Technical Information: | 1-(800)-537-8990 |
| --- | --- | --- |
| 1110 Spartan Drive | Medical Emergency: | 1-(888)-314-6171 (24 hours) |
| Maumee, OH 43537 | Chemical Leak/Spil Emergency: | CHEMTREC 1-(800) 424-9300 (24 hours) |

| Shipping Description: | Compounds, metal cutting, liquids, n.o.s. |
| --- | --- |

| NFPA Ratings: |  | HMIS Ratings: |  |
| --- | --- | --- | --- |
| Health: | 2 - Moderate | Health: | *2 - Moderate; Chronic Hazard |
| Fire: | 1- Slight | Fire: | 1- Slight |
| Reactivity: | 0 - Minimal | Reactivity: | 0 - Minimal |
|  |  | Pers. Prot. Equip.: | See Section VIII |

SECTION II: HAZARDOUS INGREDIENTS

SECTION II: HAZARDOUS INGREDIENTS
(Listed when present at 1% or greater, carcinogens at 0.1% or greater) All component chemicals are listed or exempted from listing on the "TSCA Inventory" of chemical substances maintained by the U.S. Environmental Protection Agency.

| Chemical Name(s) | % W | CAS Registry No. | Table Z- 1 - A |  |  | NTP, IARC or OSHA Carcinogen |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | TWA mg/m3 | STEL mg/m 3 | CELING mg/m3 |  |
| Hydrotreated naphthenic oil | 20-30 | 64742-52-5 | 5 (Oil mist) | Not Established | Not Established | No |
| Hydrotreated naphthenic oil | 20-30 | 64742-53-6 | 5 (Oil mist) | Not Established | Not Established | No |
| Chlorinated paraffin | 15-20 | 63449-39-8 | Not Established | Not Established | Not Established | No |
| Potassium carboxylate | 5-10 | Proprietary | Not Established | Not Established | Not Established | No |
| Alkyl succinate potassium salt | 1-5 | Proprietary | Not Established | Not Established | Not Established | No |
| Alcohol ethoxylate | 1-5 | 34398-01-1 | Not Established | Not Established | Not Established | No |
| Amine carboxylate salt | 1-5 | Proprietary | Not Established | Not Established | Not Established | No |
| Hexahydro-1 3 5-tris (2-hydroxyethyl)-s-triazine | 1-5 | 4719-04-4 | Not Established | Not Established | Not Established | No |

SECTION III: PHYSICAL DATA

SECTION III: PHYSICAL DATA

| Boiling Point: | > 300 °F | Vapor Pressure: | Unknown |
| --- | --- | --- | --- |
| Vapor Density (AIR = 1): | Unknown | Solubility in Water: | Forms an emulsion |
| pH: | 10% Solution: 10.0-10.8 | Specific Gravity (H2O=1): | 0.977 |
| Evaporation Rate (but.ace.=1): | <1 | Percent Solid by Weight: | 95-100 |
| Physical State: | Liquid |  |  |

| Appearance & Odor: | Transparent brown liquid / Mild amine odor |
| --- | --- |

SECTION IV: FIRE & EXPLOSIVE HAZARD DATA

SECTION IV: FIRE & EXPLOSIVE HAZARD DATA

| Flash Point: | > 230°F | Method Used: | Estimate |
| --- | --- | --- | --- |
| Flammable Limits: | Not Established | Flame Extension: | N/A |

| Extinguishing Media: | Foam, dry chemical, CO2, water fog or spray. |
| --- | --- |
| Special Fire Fighting Procedures: | Use air supplied breathing equipment for enclosed areas. Avoid breathing vapor or fumes. |
| Unusual Fire & Explosive Hazards: | Do not mix or store with strong oxidants like liquid chlorine or concentrated oxygen. |

SECTION V: HEALTH HAZARD DATA

SECTION V: HEALTH HAZARD DATA

| Threshold Limit Value: | Not Established | Primary Routes of Entry: | Inhalation, Skin Contact, Eyes and Oral |
| --- | --- | --- | --- |

| Effects of Overexposure- Conditions to Avoid: | CAUSES EYE IRRITATION: Symptoms may include pain, tear production and redness. CAUSES SKIN IRRITATION: Symptoms may include redness, pain and possible dermatitis. HARMFUL IF SWALLOWED: Symptoms may include stomach or intestinal upset. MAY CAUSE RESPIRATORY IRRITATION: Symptoms may include irritation of nose, throat and airways. Contains petroleum distillates which may be aspirated into the lungs during swallowing or vomiting resulting in lung inflammation and other lung injury. The effects of prolonged or repeated exposure are not well established. Repeated overexposure may cause liver and kidney effects. Avoid contact with eyes, skin and clothing. Do not taste or swallow. Avoid breathing product mist. Wash thoroughly after handling. |
| --- | --- |
| Conditions Aggravated by Use: | Use of this product may aggravate preexisting skin; eye and respiratory disorders including asthma and dermatitis. |

Emergency & First Aid Procedures:

| Eyes: | Immediately flush eyes with water for at least 15 minutes. Remove contact lenses. Get medical attention if irritation persists. |
| --- | --- |
| Skin: | Remove contaminated clothing. Wash skin thoroughly with soap and water. Get medical attention if irritation persists. Wash contaminated clothing before reuse. |
| Ingestion: | Do not induce vomiting. Call a physician immediately. |
| Inhalation: | Move person to fresh air. Get medical attention if irritation persists. |

SECTION VI: REACTIVITY DATA

SECTION VI: REACTIVITY DATA

| Stability: | Stable | Incompatible Materials: | Strong oxidizing agents |
| --- | --- | --- | --- |
| Hazardous Decomposition Products: | CO, CO2, sulfur oxides if burned | Hazardous Polymerization: | Will Not Occur |

SECTION VII: SPILL OR LEAK PROCEDURES

SECTION VII: SPILL OR LEAK PROCEDURES

| Steps to be Taken in Case Material is Released or Spilled: | Prevent liquid from entering sewers, watercourses or low areas. Contain spilled liquid with sand, earth or other inert absorbent. Do not use combustible materials such as sawdust. In addition it is recommended that local authorities be contacted to determine if there are local reporting requirements. |
| --- | --- |
| Waste Disposal Method: | Consult an expert on disposal of recovered material and ensure conformity with federal, state and local regulations. |

SECTION VIII: SPECIAL PROTECTION INFORMATION

SECTION VIII: SPECIAL PROTECTION INFORMATION

| Respiratory Protection: | Not normally required when adequate ventilation is provided. However, if exposure limits are exceeded (see Section II) or if respiratory irritation occurs, use of a NIOSH approved respirator appropriate for the conditions of use and chemicals in Section II should be considered. |
| --- | --- |
| Ventilation: | Local exhaust: Follow ACGIH Industrial ventilation recommendations. Mechanical (Recommended)- Provide ventilation suitable for type of building structure and work area. |
| Protective Gloves(Specify Type): | Rubber or other impervious gloves are recommended when handing undiluted product. Gloves are also recommended when working with the diluted product for individuals with sensitive skin. |
| Eye Protection(Specify Type): | Splash goggles are recommended to prevent eye contact. |
| Other Protective Equipment: | For further guidance see 29 CFR 1910.132-138. |

SECTION IX: SPECIAL PRECAUTIONS

SECTION IX: SPECIAL PRECAUTIONS

| Precautions; Handling & Storing: | Store in a cool dry place away from strong oxidizers and extreme heat. Keep containers sealed and stored upright when not in use. |
| --- | --- |
| Other Precautions: | Keep out of reach of children. |

| © SCC 11/30/2005 | Name: | Ronald T. Cook | Title: | Manager, Regulatory Affairs |
| --- | --- | --- | --- | --- |
| CUTTER EXP HW | Effective Date: | 11/30/2005 | Supercedes: | 06/30/2005 |
| Ref: 29 CFR 1910.1200 (OSHA) | Changes: | General Update |  |  |

This document has been prepared using data from sources considered technically reliable. It does not constitute a warranty, express or implied, as to the accuracy of the information contained herein. Actual conditions of use and handling are beyond sellers control. User is responsible to evaluate all available information when using product for any particular use and to comply with all Federal, State and Local laws and regulations.

Bottom of Form